Voice Activity Detection (VAD)

1. Title Page

• Task Name: Voice Activity Detection (VAD)

• Intern Name: Vamsi Krishna Darla

• Date of Task Given: 22/05/2026

• Supervisor: Vasudha

• Role: Developed and tested the Voice Activity Detection (VAD) engine.

2. Executive Summary

The Voice Activity Detection (VAD) system was developed as part of the Audio Processing module to identify whether speech is present in an audio stream. The engine detects speech while filtering background noise and breathing sounds using energy and frequency analysis.

Outcome of Work:

• Implemented speech activity detection using audio frame analysis

• Processed real-time audio streams

• Reduced false detections caused by noise and breathing sounds

• Performed testing under multiple audio conditions

• Improved speech segmentation accuracy

3. Task Overview

Objective

• Detect speech segments in audio input

• Distinguish speech from silence and background noise

• Improve efficiency in speech processing systems

Scope

• Audio frame processing

• Speech activity detection

• Noise filtering

• Energy and frequency feature extraction

• Testing under different noise conditions

• Performance validation

4. Technologies Used

• Python - Main programming language

• NumPy - Numerical operations and frame processing

• SciPy - Signal processing utilities

• Librosa - Audio feature extraction

• PyAudio - Real-time audio stream handling

• Matplotlib - Audio waveform visualization

• VS Code - Development environment

5. Implementation Details

The VAD engine uses an energy-threshold-based approach where audio is divided into short frames and analyzed to determine whether speech is present.

Step-by-Step Workflow

• Capture audio input from microphone or audio file

• Split audio into 20–30 ms frames

• Calculate frame energy and zero-crossing rate

• Compare extracted values with thresholds

• Classify frames as speech or silence

• Merge continuous speech segments

Edge Case Handling Implementation

To improve detection accuracy, additional filtering logic was implemented for handling breathing sounds, short noise spikes, and continuous background noise.

Breathing Sound Filtering
Breathing sounds often create low-energy audio spikes that may be incorrectly detected as speech. To reduce false positives, minimum speech duration filtering was applied.

MIN_SPEECH_FRAMES = 5
speech_frames = 0

if energy > ENERGY_THRESHOLD:
    speech_frames += 1
else:
    speech_frames = 0

if speech_frames >= MIN_SPEECH_FRAMES:
    speech_detected = True

This logic ensures that only continuous voice frames are classified as speech while ignoring short breathing sounds.

Background Noise Handling
Background noise was controlled using threshold-based filtering and Zero Crossing Rate (ZCR) analysis.

LOW_ZCR_THRESHOLD = 0.02

if zcr < LOW_ZCR_THRESHOLD:
    ignore_frame = True

The system ignores low-frequency environmental noise and improves speech detection stability in medium-noise environments.

Noise Spike Reduction
Sudden short-duration sound spikes were filtered by rejecting isolated frames that did not meet minimum duration requirements.

if frame_energy < NOISE_THRESHOLD:
    speech_detected = False

This reduced false activation caused by keyboard taps, microphone disturbances, and environmental sounds.

• Filter short noise spikes and breathing sounds using minimum speech duration and threshold filtering

6. Testing & Validation

• Unit testing for frame energy calculations

• Background noise testing

• Validation using recorded speech samples

• Low-volume speech testing

• False-positive testing using breathing sounds

Constraints

• Requires proper microphone input

• Threshold tuning needed for different environments

• Sensitive to excessive environmental noise

Edge Cases:

• Continuous background noise

• Very low-volume speech

• Sudden sound spikes

• Breathing sounds detected as speech → Implemented minimum speech duration filtering and ZCR-based noise rejection

7. Results & Outcome

• Successfully detected speech segments in audio streams

• Reduced false detection caused by breathing sounds

• Improved noise handling using threshold tuning

• Maintained stable performance in medium-noise environments

• Achieved accurate real-time voice activity detection

8. Challenges & Solutions

• Background noise interference → Applied threshold tuning and filtering

• Breathing sounds detected as speech → Implemented minimum speech duration filtering and ZCR-based noise rejection

• Low-volume voice detection → Adjusted energy sensitivity dynamically

• Real-time processing delays → Optimized frame size and processing speed

9. Conclusion

The Voice Activity Detection engine successfully identified speech activity while minimizing noise interference. This task improved practical knowledge in audio signal processing, real-time audio systems, debugging, and speech analysis.

10. References

• Python Official Documentation

• Librosa Documentation

• NumPy Documentation

• SciPy Signal Processing Documentation

• Voice Activity Detection Research Papers and Tutorials

Repository

GitHub Repository Link :

https://github.com/vamsikrishna78425/Task--1---VamsiKrishna-VAD-engine

Final Submission

Deadline: 28/05/2026

Format: Word Document (.docx)

Submission To: Ayesha faquih